# Narmada Watershed Delineation and Morphometric Analysis using QGIS

Author: Priyanka
Software: QGIS 3.44, GDAL

# 1. Introduction

This project delineates the Narmada watershed using a DEM and performs hydrological, terrain, vegetation, land use/land cover, and morphometric analyses using open-source GIS tools.

# 2. Study Area

Study Area: Narmada Watershed
Projection: WGS 84 / UTM Zone 44N (EPSG:32644).

# 3. Data Used

| Dataset | Source | Resolution |
| --- | --- | --- |
| SRTM DEM | USGS | 30 m |
| Landsat 8 Surface Reflectance | USGS EarthExplorer | 30 m |
| ESA WorldCover | ESA | 10 m |

# 4. Methodology

The workflow included DEM preprocessing, sink filling, flow direction and accumulation, stream extraction, watershed delineation, slope, hillshade and contour generation, NDVI from Landsat 8, clipping ESA WorldCover to the watershed, and morphometric analysis.

# 5. Results

## Watershed Boundary

## DEM

## Slope

## Stream Network

## Contour Map

## NDVI

## Land Use/Land Cover

# 6. Morphometric Analysis

| Parameter | Value |
| --- | --- |
| Watershed Area | 1187.59 km² |
| Perimeter | 367.14 km |
| Maximum Elevation | 641.10 m |
| Minimum Elevation | 330.70 m |
| Relief | 310.40 m |
| Total Stream Length | 1062.43 km |
| Stream Segments | 1639 |
| Drainage Density | 0.895 km/km² |
| Stream Frequency | 1.38 streams/km² |

The watershed exhibits a moderate drainage density and stream frequency, indicating a well-developed drainage network. Elevation and slope maps reveal terrain variability, while NDVI and ESA WorldCover provide insight into vegetation and land cover patterns.

# LULC Statistics

| DN | Class | Area (km²) | Percent |
| --- | --- | --- | --- |
| 10 | Tree Cover | 262.11 | 22% |
| 20 | Shrubland | 18.70 | 2% |
| 30 | Grassland | 241.27 | 20% |
| 40 | Cropland | 613.10 | 52% |
| 50 | Built-up | 32.85 | 3% |
| 60 | Bare/Sparse Vegetation | 5.95 | 1% |
| 80 | Water Bodies | 13.40 | 1% |
| 90 | Herbaceous Wetland | 0.02 | 0% |

# 7. Discussion

The integrated analysis demonstrates the usefulness of DEM-based hydrological modeling combined with remote sensing products for watershed characterization and resource management.

# 8. Conclusion

The project successfully delineated the watershed, generated terrain and hydrological products, assessed vegetation using NDVI, mapped land use/land cover using ESA WorldCover, and computed key morphometric parameters.

# 9. References

1. QGIS Documentation.
2. USGS EarthExplorer.
3. ESA WorldCover User Manual.
4. NASA SRTM DEM Documentation.